MODULAR GNSS GEOFENCING SYSTEM

COMPLETE HARDWARE IMPLEMENTATION REFERENCE

3-Device Architecture: Platform Module · Ground Control Unit · GPS Spoofer/Validator

| Prepared by | Asad Sayyed |
| --- | --- |
| Institution | Vishwakarma Institute of Technology, Pune |
| Document Type | Hardware Implementation Reference — Wiring Diagrams, Datasheets & Assembly |
| Date | April 2026 |
| Revision | 1.0 — Complete Hardware Edition |

# TABLE OF CONTENTS

# 1. SYSTEM OVERVIEW & HARDWARE ARCHITECTURE

This document provides the complete hardware implementation of the 3-Device Modular GNSS Geofencing System. It covers every component's wiring connections, pin assignments, datasheet parameters, power distribution, and physical assembly notes. This is the definitive hardware reference for building, testing, and troubleshooting all three devices.

The Three Devices:

| Device | Name | Form Factor | Core MCU | Primary Role |
| --- | --- | --- | --- | --- |
| Device 1 (PM) | Platform Module | ~100×80mm PCB/enclosure | ESP32 DevKit | Sense, navigate, enforce geofence, stream video |
| Device 2 (GCU) | Ground Control Unit | Handheld box + laptop | ESP32 DevKit + Arduino Nano | Send commands, receive telemetry, display data |
| Device 3 (SV) | GPS Spoofer/Validator | Small standalone PCB | ESP32 DevKit | Inject synthetic GPS to validate geofence response |

## 1.1 Inter-Device Communication Links

| Link | Technology | Frequency / Speed | Range | Data |
| --- | --- | --- | --- | --- |
| PM ↔ GCU | LoRa SX1278 | 433 MHz / SF9 BW125 | 2–3 km LOS | Telemetry, commands, geofence |
| GCU ↔ Laptop | USB UART / HC-05 BT | 115200 baud / BT2.0 | Wired / <10 m BT | JSON telemetry stream, commands |
| PM → Laptop Video | ESP32-CAM WiFi AP | 2.4 GHz 802.11b/g/n | ~100 m LOS | MJPEG video frames |
| SV → PM (test) | UART NMEA Injection | 9600 baud TTL UART | Direct wire only | Synthetic NMEA sentences |

## 1.2 System Power Architecture

All three devices are powered from a 5V supply. The ESP32 on-board AMS1117-3.3V regulator converts 5V to 3.3V for all logic-level sensors. The RCWL-0516 and ESP32-CAM require direct 5V.

| Device | Input Voltage | Avg Current Draw | Peak Current | Recommended Supply |
| --- | --- | --- | --- | --- |
| Device 1 — PM | 5V DC | ~370 mA | ~830 mA | 5V / 2A regulated (USB-C or BEC from UAV) |
| Device 2 — GCU | 5V DC (USB) | ~120 mA | ~200 mA | 5V USB from laptop or 18650 power bank |
| Device 3 — SV | 5V DC | ~80 mA | ~120 mA | 5V USB power bank or 3× AA batteries + 7805 |

⚠  Always measure the 3.3V rail with a multimeter before connecting sensors. Low-quality USB cables can cause voltage sag under load. Use at minimum a 2A capable 5V supply for the Platform Module.

# 2. COMPONENT DATASHEETS — ALL SENSORS & MODULES

2.1  NEO-6M GPS Module

| Parameter | Specification |
| --- | --- |
| Manufacturer / Chipset | u-blox AG — NEO-6M-0-001 |
| GNSS System | GPS L1 C/A, 1575.42 MHz |
| Channels | 50 parallel tracking channels |
| Position Accuracy (CEP) | 2.5 m open sky (no SBAS) |
| Update Rate | 1 Hz default; configurable to 5 Hz via UBX-CFG-RATE |
| Cold Start TTFF | ~27 seconds (no almanac stored) |
| Hot Start TTFF | < 1 second (valid almanac + ephemeris in battery-backed RAM) |
| Sensitivity — Tracking | -161 dBm |
| Sensitivity — Acquisition | -148 dBm |
| Operating Voltage (VCC) | 3.3V (module has onboard 3.3V LDO — accepts 2.7V–3.6V) |
| Operating Current | 45 mA (navigation); 11 mA (power save mode) |
| Logic Level (TX/RX) | 3.3V TTL UART (NOT 5V tolerant on RX pin) |
| Default UART Baud Rate | 9600 bps 8-N-1 (reconfigurable via UBX-CFG-PRT) |
| Output Format | NMEA 0183 v3.01: GPGGA, GPGLL, GPRMC, GPVTG, GPGSV, GPGSA |
| PPS Output | 1 pulse-per-second, 100ms wide, 3.3V TTL (accuracy ±30 ns with fix) |
| Antenna Connector | U.FL / IPEX with passive ceramic patch antenna included |
| Operating Temperature | -40°C to +85°C |
| Dimensions (module PCB) | 25.1 × 35.4 mm (NEO-6M breakout board) |
| RoHS Compliant | Yes |
| Backup Battery | Onboard LIR2032 button cell — retains almanac & RTC |
| Reference Document | u-blox NEO-6M Hardware Integration Manual, GPS-13C Rev.1 |

The NEO-6M's UART RX pin accepts UBX binary commands to configure update rate, disable unwanted NMEA sentences, and enable Software Defined Mode for NMEA injection (Device 3). Connect a 1kΩ resistor in series with ESP32 TX → NEO-6M RX if level mismatch is suspected.

2.2  MPU-6050 IMU (GY-521 Breakout)

| Parameter | Specification |
| --- | --- |
| Manufacturer / Chipset | InvenSense Inc. — MPU-6050 |
| Sensors Integrated | 3-axis accelerometer + 3-axis gyroscope + temperature sensor (DMP) |
| Accelerometer Full-Scale | ±2g / ±4g / ±8g / ±16g (configurable, AFS_SEL register 0x1C) |
| Accelerometer Sensitivity | 16384 LSB/g (±2g), 8192 LSB/g (±4g), 4096 LSB/g (±8g) |
| Gyroscope Full-Scale | ±250 / ±500 / ±1000 / ±2000 °/s (FS_SEL register 0x1B) |
| Gyroscope Sensitivity | 131 LSB/°/s (±250), 65.5 LSB/°/s (±500) |
| ADC Resolution | 16-bit signed for each axis (both accel and gyro) |
| Internal Sample Rate | Up to 8 kHz (SMPLRT_DIV register divides this) |
| DLPF (Digital Low-Pass Filter) | Register 0x1A — value 0x03 = 44 Hz cutoff (recommended for UAV) |
| Interface | I2C (up to 400 kHz Fast Mode) — primary interface |
| I2C Address | 0x68 (AD0 pin LOW, default); 0x69 (AD0 pin HIGH) |
| Supply Voltage (VDD) | 2.375V – 3.46V (logic 3.3V; GY-521 breakout has onboard 3.3V regulator — accepts 3.3V–5V) |
| Operating Current | 3.9 mA (normal); 5 µA (sleep mode) |
| Logic Level (SDA/SCL) | 3.3V — GY-521 breakout includes 4.7kΩ pull-ups to 3.3V |
| Temperature Sensor | Accuracy ±1°C, output in raw 16-bit; formula: Temp°C = (raw/340.0) + 36.53 |
| Interrupt Pin | INT — active HIGH, configurable for data ready / motion detect |
| Operating Temperature | -40°C to +85°C |
| Dimensions (GY-521 board) | 20.3 × 15.6 mm |
| Recommended Accel Scale (PM) | ±4g — 8192 LSB/g (best for vehicle dynamics without saturation) |
| Recommended Gyro Scale (PM) | ±500 °/s — handles fast maneuvers without saturation |
| Reference Document | InvenSense MPU-6050 Product Specification Rev. 3.4 (2013) |

Place a 100nF decoupling capacitor between VCC and GND as close to the MPU-6050 as possible. Keep I2C traces short (<5 cm). If sharing I2C bus with BMP280, verify no address conflicts (0x68 vs 0x76 — no conflict).

2.3  GY-BMP280 Barometric Pressure/Altitude Sensor

| Parameter | Specification |
| --- | --- |
| Manufacturer / Chipset | Bosch Sensortec GmbH — BMP280 |
| Measurement Type | Absolute barometric pressure + temperature |
| Pressure Range | 300 hPa to 1100 hPa (equivalent to -500 m to +9000 m altitude) |
| Pressure Resolution | 0.0016 hPa (≈ 0.13 m altitude resolution at oversampling ×16) |
| Pressure Absolute Accuracy | ±1 hPa (≈ ±8 m typical) |
| Pressure Relative Accuracy | ±0.12 hPa (≈ ±1 m over 1 hour — excellent for hold/hover) |
| Temperature Range | -40°C to +85°C |
| Temperature Resolution | 0.01°C |
| Interface Options | I2C (up to 3.4 MHz) or SPI (up to 10 MHz) |
| I2C Address | 0x76 (SDO/CSB pin LOW/GND); 0x77 (SDO pin HIGH/VCC) |
| Supply Voltage (VDD) | 1.71V – 3.6V (GY-BMP280 breakout accepts 3.3V–5V via onboard regulator) |
| Operating Current | 2.74 mA (ultra high-res mode); 0.1 µA (sleep) |
| Recommended Mode for PM | osrs_p=8× oversampling, osrs_t=1×, IIR filter coeff=8, ~25 Hz ODR |
| IIR Filter | Coefficients: 2, 4, 8, 16 — reduces short-term pressure noise |
| Output Data Rate | Up to 157.5 Hz (low-power mode); ~25 Hz in recommended config |
| Altitude Formula | Alt = 44330 × (1 - (P / P0)^0.1903) — calibrate P0 at launch |
| Dimensions (GY board) | 11 × 11 mm (bare IC); ~15 × 11 mm breakout |
| Reference Document | Bosch BMP280 Datasheet v1.27 (2019) |

Record the BMP280 pressure reading at launch (P0) to use as the altitude reference. Altitude changes of less than 1 m should use the IIR-filtered output. Shield from direct airflow / prop wash as dynamic pressure causes false altitude readings.

2.4  RCWL-0516 Microwave Doppler Radar Sensor

| Parameter | Specification |
| --- | --- |
| Manufacturer | RCW Technology Co., Ltd. |
| Sensing Technology | 3.18 GHz Doppler Microwave Radar — EIRP ~26.4 dBm |
| Detection Principle | Doppler effect — detects movement of objects (human, animal, vehicle) |
| Detection Range | 5–7 m typical; 5 m reliable in most conditions |
| Detection Angle | Omnidirectional (~360°) — use copper foil shielding to limit to forward arc |
| Output Type | Digital — HIGH (+3.3V) when motion detected; LOW otherwise |
| Output Hold Time | ~2 seconds after last motion event (capacitor C-TM controls this) |
| Supply Voltage | 4V – 28V (optimum at 5V; 3.3V also works with reduced range) |
| Supply Current | 2.8 mA typical at 5V |
| Output Logic Level | 0V / 3.3V (direct connection to ESP32 GPIO — no level shifting needed) |
| PCB Dimensions | 17.4 × 17.1 mm |
| Sensitivity Adjustment | R-GN resistor pad — add/remove to adjust detection threshold |
| Repeat Trigger Mode | Default: H level always high when motion present; configurable via RCWL-0516 pads |
| Known Interference Sources | Fast-spinning propellers, fluorescent lighting (50/60 Hz), RF noise |
| Operating Temperature | -20°C to +80°C |
| Reference Document | RCWL-0516 Microwave Sensor Module Datasheet, RCW Technology Co. |

⚠  RCWL-0516 will false-trigger on multi-rotor drone propellers. Mount it in the nose (forward-facing) and shield three sides with copper tape or aluminum foil to create a directional forward cone. Use a 5-sample software debounce window in firmware.

2.5  SX1278 RA-02 LoRa Transceiver Module (433 MHz)

| Parameter | Specification |
| --- | --- |
| Manufacturer / Chipset | Semtech Corporation — SX1278 (module: AI-Thinker RA-02) |
| Frequency Range | 137 MHz – 525 MHz (configured at 433.0 MHz for ISM band) |
| Modulation | LoRa (CSS — Chirp Spread Spectrum) + optional FSK/OOK |
| Spreading Factor | SF6–SF12 (configured at SF9 for 1–2 km balanced range/speed) |
| Bandwidth Options | 7.8 / 10.4 / 15.6 / 20.8 / 31.25 / 41.7 / 62.5 / 125 / 250 / 500 kHz |
| Configured Bandwidth | 125 kHz (best noise immunity) |
| Coding Rate | 4/5 to 4/8 (configured at 4/6 for field use error correction) |
| Max TX Power | +20 dBm (100 mW) — configured at +17 dBm (~50 mW) for battery life |
| RX Sensitivity | -129 dBm at SF9/BW125 (excellent range performance) |
| Data Rate at SF9/BW125 | ~1.76 kbps raw (sufficient for 24-byte telemetry at 4 Hz) |
| Interface | SPI (up to 10 MHz) — MOSI, MISO, SCK, NSS (CS), DIO0, RST |
| Supply Voltage | 3.3V (NOT 5V tolerant — use 3.3V supply only) |
| TX Current | 120 mA at +20 dBm; 87 mA at +17 dBm |
| RX Current | 9.9 mA standby; ~14 mA receive |
| Antenna Connector | U.FL / IPEX onboard; use 433 MHz quarter-wave whip (~17 cm) |
| Sync Word | Configurable (0xA5 used in this system to filter other LoRa devices) |
| Link Budget (system config) | 150 dB — theoretical max range ~5.6 km open field |
| Air Time (24-byte packet, SF9) | ~65 ms per packet |
| Module Dimensions (RA-02) | 17 × 16 mm |
| Reference Document | Semtech SX1276/77/78/79 Datasheet Rev 7 (2019) |

The SX1278 RA-02 module operates at 3.3V ONLY. All SPI lines must be 3.3V. ESP32 GPIOs are 3.3V — direct connection is safe. Do NOT connect to 5V Arduino GPIO without level shifting. Antenna must be vertical for omnidirectional coverage.

2.6  ESP32 DevKit V1 (38-pin) — Main MCU (All Three Devices)

| Parameter | Specification |
| --- | --- |
| Manufacturer / Chipset | Espressif Systems — ESP32-D0WDQ6 (dual-core) |
| CPU | Dual-core Xtensa LX6, 240 MHz maximum |
| RAM | 520 KB SRAM; 4 MB Flash (typical DevKit) |
| WiFi | 802.11 b/g/n 2.4 GHz, integrated antenna |
| Bluetooth | BT 4.2 Classic + BLE 4.2 |
| GPIO Count | 34 programmable GPIOs (38-pin DevKit) |
| ADC | 12-bit SAR ADC, 18 channels (12 available simultaneously) |
| DAC | 8-bit DAC on GPIO 25, GPIO 26 |
| UART | 3× hardware UART (UART0/1/2) |
| SPI | 4× SPI (SPI0/1 internal flash; SPI2 = HSPI; SPI3 = VSPI for user) |
| I2C | 2× I2C (software configurable on any GPIO) |
| PWM | 16 channels, 1 Hz to 40 MHz |
| Supply Voltage (VIN) | 5V on VIN pin (onboard AMS1117-3.3 regulator) |
| 3.3V Output Current | Up to 600 mA from onboard 3.3V pin (AMS1117 limit) |
| GPIO Logic Level | 3.3V (NOT 5V tolerant) |
| Input-Only GPIOs | GPIO 34, 35, 36 (VP), 39 (VN) — NO internal pull-ups |
| Operating Temperature | -40°C to +85°C |
| Current Draw | 80 mA idle; 240 mA WiFi TX burst; 5 µA deep sleep |
| Module Dimensions (DevKit) | 51.4 × 28.2 mm (38-pin DIP breakout) |
| Reference Document | Espressif ESP32 Technical Reference Manual v5.2 (2024) |

⚠  GPIO 34, 35, 36, 39 are input-ONLY — they have NO pull-up resistors and cannot source current. Connect RCWL-0516 OUT, analog reads, and other inputs here. Never connect LEDs, outputs, or SPI CS to these pins.

2.7  ESP32-CAM (OV2640) — Video Stream Module

| Parameter | Specification |
| --- | --- |
| Module | AI-Thinker ESP32-CAM (ESP32-S with OV2640 camera) |
| Camera Sensor | OV2640 — 2 Megapixel CMOS (1600×1200 UXGA) |
| Video Resolution Options | QQVGA (160×120) to UXGA (1600×1200), MJPEG format |
| Recommended Stream Res. | VGA (640×480) or SVGA (800×600) at 15–30 fps via WiFi |
| WiFi Mode Used | AP (Access Point) mode — laptop connects directly to ESP32-CAM SSID |
| MJPEG Stream URL | http://192.168.4.1/stream (default when in AP mode) |
| Supply Voltage | 5V on 5V pin (3.3V pin available for logic) |
| Supply Current | 180 mA streaming; 310 mA peak (USB or 5V BEC required — NOT 3.3V) |
| Flash LED | Onboard white LED on GPIO 4 (also used by SD — conflict — disable flash) |
| MicroSD Slot | Onboard (shares SPI with camera — NOT used in this system; PM has own SD) |
| Antenna | Onboard PCB antenna (or U.FL connector on some variants) |
| Programming Interface | Serial via FTDI/CH340 — requires external USB-Serial adapter |
| GPIO Available | Limited — IO0, IO2, IO4, IO12–IO16 (most consumed by camera/SD) |
| Dimensions | 40 × 27 mm |
| Operating Temperature | -20°C to +85°C |
| Firmware | Arduino ESP32 Camera Web Server example (AI-Thinker config) |

The ESP32-CAM is programmed and powered separately from the main ESP32 DevKit. It creates its own WiFi Access Point (e.g., SSID: 'GeoFence-Cam'). The operator's laptop connects to this SSID and opens the stream URL in a browser. No additional wiring to PM's ESP32 is required — it operates independently.

2.8  MicroSD SPI Adapter

| Parameter | Specification |
| --- | --- |
| Type | MicroSD to SPI breakout board (level-shifted version) |
| Interface | SPI (MOSI, MISO, SCK, CS) — shares VSPI bus with LoRa (separate CS) |
| Compatible SD Types | microSDHC cards up to 32 GB (FAT32 formatted) |
| Recommended Card | SanDisk 8 GB Class 10 microSDHC (write speed ≥ 10 MB/s) |
| Supply Voltage | 3.3V (level-shifted adapter) or 5V (raw SD via resistor dividers) |
| Current Draw | 25 mA typical write; 100 mA peak during write burst |
| Max SPI Speed | 25 MHz (SD spec); use 4–8 MHz for reliability on breadboard/PCB |
| File System | FAT32 — use Arduino SD.h library (included with ESP32 Arduino core) |
| Log File Format | CSV — one row per 500 ms: timestamp, lat, lon, alt, heading, roll, pitch, sats, obstacle, battery |

2.9  HC-05 Bluetooth Serial Module

| Parameter | Specification |
| --- | --- |
| Chipset | CSR BC417 — Bluetooth 2.0 + EDR Classic |
| Bluetooth Version | BT 2.0 + EDR (Enhanced Data Rate) — SPP (Serial Port Profile) |
| Range | Up to 10 m (Class 2 power) |
| Interface | UART TTL — TX, RX (and KEY/EN for AT command mode) |
| Default Baud Rate | 9600 bps (AT command changeable to 115200 bps recommended) |
| AT Command Mode | Set baud to 38400, hold KEY pin HIGH before power-on |
| Supply Voltage | 3.6V – 6V (onboard regulator; 5V direct from ESP32 5V pin is fine) |
| TX/RX Logic Level | 3.3V — do NOT connect 5V to RX pin directly; use 1kΩ + 2kΩ divider |
| Pairing PIN | Default: 1234 (or 0000) |
| Current Draw | ~35 mA connected; 8 mA standby |
| Dimensions | 26.9 × 13 mm |
| Usage in this System | GCU ↔ Laptop wireless bridge (alternative to USB cable); also used for PM BT config |

Change HC-05 baud rate to 115200 using AT commands before deployment: AT+UART=115200,0,0. The ESP32 UART level (3.3V) requires a voltage divider on the HC-05 RX pin: connect ESP32-TX → 1kΩ → HC-05-RX, and 2kΩ from HC-05-RX to GND.

2.10  Arduino Nano (ATmega328P) — GCU Display Controller

| Parameter | Specification |
| --- | --- |
| Chipset | Microchip ATmega328P-AU @ 16 MHz (5V version) |
| Flash | 32 KB (2 KB bootloader) |
| SRAM | 2 KB |
| Interface to ESP32 | UART Serial (5V TX → 1kΩ → 3.3V ESP32 RX; 3.3V ESP32 TX → Nano RX direct) |
| I2C | A4 (SDA), A5 (SCL) — drives SSD1306 OLED at 3.3V–5V |
| Supply Voltage | 5V via USB or VIN (7V–12V raw) |
| GPIO Logic Level | 5V — use level shifter or resistor divider for 3.3V devices |
| Usage in GCU | Receives display data over UART from ESP32; drives SSD1306 OLED; reads joystick ADC |
| Purpose | Offloads display and ADC tasks from ESP32 so ESP32 can focus on LoRa comms |

2.11  0.96" SSD1306 OLED Display

| Parameter | Specification |
| --- | --- |
| Controller IC | Solomon Systech SSD1306 |
| Display Size | 0.96 inch diagonal |
| Resolution | 128 × 64 pixels (monochrome) |
| Interface | I2C (default) — address 0x3C (or 0x3D if SA0 high) |
| Supply Voltage | 3.3V – 5V (onboard 3.3V regulator on most breakout boards) |
| Current Draw | ~20 mA full display on; 0.5 µA sleep |
| Library | Adafruit SSD1306 + Adafruit GFX (Arduino Library Manager) |
| Usage on GCU | Displays: GPS lat/lon, altitude, RSSI, geofence status, battery %, satellite count |

2.12  KY-023 Analog 2-Axis Joystick Module

| Parameter | Specification |
| --- | --- |
| Type | 2-axis potentiometer joystick with center push button |
| Output | Analog 0–VCC on VRX (X-axis) and VRY (Y-axis); Digital SW (button, active LOW) |
| Center Position Output | ~VCC/2 = ~1.65V (on 3.3V supply) or ~2.5V (5V supply) |
| Supply Voltage | 3.3V or 5V (connect to 3.3V on ESP32 for direct ADC reading) |
| ADC Connections | VRX → ESP32 GPIO32 (ADC1_CH4); VRY → ESP32 GPIO33 (ADC1_CH5) |
| ADC Resolution | 12-bit on ESP32 (0–4095 range; ~2048 at center) |
| Usage on GCU | Manual override commands — X: left/right heading, Y: forward/backward speed |

# 3. DEVICE 1 — PLATFORM MODULE (PM): COMPLETE WIRING

The Platform Module is the core sensing and navigation payload. It runs on a single ESP32 DevKit and interfaces with 7 sensors/modules over UART, I2C, SPI, and Digital GPIO. All sensors except the RCWL-0516 and ESP32-CAM run from 3.3V.

## 3.1 PM Master Pin Allocation Table

| GPIO Pin | Direction | Connected To | Signal / Protocol | Voltage | Notes |
| --- | --- | --- | --- | --- | --- |
| GPIO 16 (RX2) | INPUT | NEO-6M TX (yellow) | UART2 — NMEA 9600 baud | 3.3V TTL | HardwareSerial2 in firmware |
| GPIO 17 (TX2) | OUTPUT | NEO-6M RX (white) | UART2 — UBX config cmds | 3.3V TTL | Optional; configure at boot |
| GPIO 21 (SDA) | BIDIR | MPU-6050 SDA + BMP280 SDA | I2C data (shared bus) | 3.3V | 4.7kΩ pull-up to 3.3V — ONE resistor for both |
| GPIO 22 (SCL) | OUTPUT | MPU-6050 SCL + BMP280 SCL | I2C clock (shared bus) | 3.3V | 4.7kΩ pull-up to 3.3V — ONE resistor for both |
| GPIO 18 (SCK) | OUTPUT | LoRa SCK + SD SCK | SPI VSPI clock (shared) | 3.3V | LoRa and SD share VSPI bus |
| GPIO 19 (MISO) | INPUT | LoRa MISO + SD MISO | SPI VSPI data in | 3.3V | Shared VSPI bus |
| GPIO 23 (MOSI) | OUTPUT | LoRa MOSI + SD MOSI | SPI VSPI data out | 3.3V | Shared VSPI bus |
| GPIO 5 | OUTPUT | LoRa NSS/CS | SPI chip select (active LOW) | 3.3V | Drive LOW to select LoRa |
| GPIO 15 | OUTPUT | MicroSD CS | SPI chip select (active LOW) | 3.3V | Drive LOW to select SD |
| GPIO 2 | INPUT | LoRa DIO0 | TX/RX done interrupt | 3.3V | Required for RadioLib interrupt mode |
| GPIO 4 | OUTPUT | LoRa RST | LoRa hardware reset (LOW = reset) | 3.3V | Drive LOW 10ms then HIGH to reset |
| GPIO 34 | INPUT ONLY | RCWL-0516 OUT | Digital HIGH = obstacle detected | 3.3V | Input-only pin — no internal pull-up available |
| GPIO 26 | OUTPUT | Status LED anode (via 330Ω) | Visual indicator | 3.3V | 330Ω series resistor to GND |
| GPIO 1 (TX0) | OUTPUT | USB Serial / HC-05 RX | UART0 debug / BT config | 3.3V | Shared with USB-Serial on DevKit |
| GPIO 3 (RX0) | INPUT | USB Serial / HC-05 TX | UART0 debug / BT config | 3.3V | Avoid using during USB debug |
| 3V3 pin | POWER OUT | NEO-6M VCC, MPU VCC, BMP VCC, LoRa VCC, SD VCC | 3.3V supply rail | 3.3V | Max 600 mA total from AMS1117 |
| 5V (VIN) pin | POWER IN | Vehicle 5V supply → VIN | Main 5V input | 5V | Or USB 5V from laptop during dev |
| GND | GROUND | All module GND pins | Common ground | 0V | Star-topology GND recommended |
| RCWL-0516 VCC | POWER OUT | ESP32 VIN/5V pin → RCWL VCC | 5V supply for radar | 5V | RCWL requires 5V — connect to DevKit 5V/VIN pin |
| ESP32-CAM 5V | POWER OUT | Separate 5V rail → CAM 5V | 5V for camera module | 5V | CAM draws 180–310 mA — use dedicated supply if possible |

## 3.2 PM Wiring Diagram — ASCII Schematic

The following diagram shows all connections for Device 1. Power rails are shown on the left; signal connections on the right.

╔══════════════════════════════════════════════════════════════════════════════╗

║           DEVICE 1 — PLATFORM MODULE (PM) — COMPLETE WIRING DIAGRAM        ║

╚══════════════════════════════════════════════════════════════════════════════╝

POWER INPUT                    ESP32 DevKit (38-pin)

─────────                 ┌──────────────────────────┐

5V @ 2A ──────── VIN ──── │ VIN          GPIO16(RX2) │──── NEO-6M TX  (NMEA out)

GND ──── │ GND          GPIO17(TX2) │──── NEO-6M RX  (UBX config)

│                          │

3.3V RAIL ─────────────── │ 3V3    [I2C BUS SHARED]  │

(from AMS1117)            │          GPIO21(SDA) ────┼──┬─ MPU-6050 SDA

│                          │  └─ BMP280  SDA

│          GPIO22(SCL) ────┼──┬─ MPU-6050 SCL

│                          │  └─ BMP280  SCL

│                          │   (4.7kΩ pull-up × 2 to 3.3V)

│                          │

│    [SPI VSPI BUS SHARED] │

│          GPIO18(SCK) ────┼──┬─ LoRa SCK

│                          │  └─ SD  SCK

│         GPIO19(MISO) ────┼──┬─ LoRa MISO

│                          │  └─ SD  MISO

│         GPIO23(MOSI) ────┼──┬─ LoRa MOSI

│                          │  └─ SD  MOSI

│           GPIO5(CS)  ────┼──── LoRa NSS/CS (chip select)

│          GPIO15(CS)  ────┼──── SD  CS     (chip select)

│           GPIO2      ────┼──── LoRa DIO0  (IRQ)

│           GPIO4      ────┼──── LoRa RST

│                          │

│           GPIO34     ────┼──── RCWL-0516 OUT (obstacle)

│           GPIO26     ────┼──── 330Ω ── LED(+) ── GND

│           GPIO1(TX0) ────┼──── HC-05 RX  (via 1kΩ divider)

│           GPIO3(RX0) ────┼──── HC-05 TX  (direct)

└──────────────────────────┘

╔══════════ MODULE POWER CONNECTIONS ═══════════╗

║  NEO-6M   VCC ─── 3.3V   GND ─── GND         ║

║  MPU-6050 VCC ─── 3.3V   GND ─── GND         ║

║  BMP280   VCC ─── 3.3V   GND ─── GND         ║

║  LoRa RA-02  VCC ─── 3.3V   GND ─── GND      ║

║  MicroSD  VCC ─── 3.3V   GND ─── GND         ║

║  RCWL-0516   VCC ─── 5V(VIN) GND ─── GND     ║

║  ESP32-CAM   5V ─── 5V(VIN)  GND ─── GND     ║

╚═══════════════════════════════════════════════╝

I2C SHARED BUS DETAIL:

ESP32 3.3V ─── 4.7kΩ ─┬─ SDA ─── GPIO21 ─── MPU-6050(SDA) ─── BMP280(SDA)

ESP32 3.3V ─── 4.7kΩ ─┴─ SCL ─── GPIO22 ─── MPU-6050(SCL) ─── BMP280(SCL)

(Only ONE set of pull-ups needed; GY-521 & GY-BMP280 have onboard resistors — remove one)

SPI SHARED BUS DETAIL:

ESP32 GPIO18 (SCK)  ─────────┬────────── LoRa SCK  ─── SD SCK

ESP32 GPIO19 (MISO) ─────────┼────────── LoRa MISO ─── SD MISO

ESP32 GPIO23 (MOSI) ─────────┴────────── LoRa MOSI ─── SD MOSI

ESP32 GPIO5         ──── LoRa CS only   (active LOW pulls LoRa; SD CS stays HIGH)

ESP32 GPIO15        ──── SD CS only     (active LOW pulls SD; LoRa CS stays HIGH)

## 3.3 PM Individual Module Wiring Tables

NEO-6M GPS Module:

| NEO-6M Pin | Connect To | Wire Color (suggested) | Notes |
| --- | --- | --- | --- |
| VCC | ESP32 3V3 | Red | 3.3V power |
| GND | ESP32 GND | Black | Common ground |
| TX | ESP32 GPIO 16 (RX2) | Yellow | NMEA sentences out → ESP32 receive |
| RX | ESP32 GPIO 17 (TX2) | White | UBX config commands in |
| PPS | Not connected (optional) | — | 1 Hz pulse — can connect to any input GPIO for timing |

MPU-6050 IMU (GY-521 Breakout):

| MPU-6050 Pin | Connect To | Notes |
| --- | --- | --- |
| VCC | ESP32 3V3 | 3.3V (GY-521 has regulator — also accepts 5V, but use 3.3V here) |
| GND | ESP32 GND | Common ground |
| SDA | ESP32 GPIO 21 (SDA) | I2C data — shared with BMP280 |
| SCL | ESP32 GPIO 22 (SCL) | I2C clock — shared with BMP280 |
| AD0 | GND | Keeps I2C address at 0x68 (leave floating = 0x68 default) |
| INT | Not connected (optional) | Interrupt for data ready — can connect to any GPIO if needed |
| XDA/XCL | Not connected | Auxiliary I2C for magnetometer — unused |

GY-BMP280 Barometric Sensor:

| BMP280 Pin | Connect To | Notes |
| --- | --- | --- |
| VCC (3.3V) | ESP32 3V3 | 3.3V power |
| GND | ESP32 GND | Common ground |
| SDA | ESP32 GPIO 21 (SDA) | I2C data — shared with MPU-6050 (same wire) |
| SCL | ESP32 GPIO 22 (SCL) | I2C clock — shared with MPU-6050 (same wire) |
| CSB/CS | 3.3V (VCC) | Pull HIGH to select I2C mode (not SPI) |
| SDO | GND | Sets I2C address to 0x76 (tie to VCC for 0x77) |

LoRa SX1278 RA-02 Module:

| LoRa Pin | Connect To | Notes |
| --- | --- | --- |
| VCC | ESP32 3V3 | MUST be 3.3V — do NOT use 5V |
| GND | ESP32 GND | Common ground |
| SCK | ESP32 GPIO 18 (SCK) | SPI VSPI clock |
| MISO | ESP32 GPIO 19 (MISO) | SPI data out from LoRa |
| MOSI | ESP32 GPIO 23 (MOSI) | SPI data in to LoRa |
| NSS/CS | ESP32 GPIO 5 | Active LOW chip select — pulled LOW by ESP32 during SPI transaction |
| DIO0 | ESP32 GPIO 2 | Interrupt — HIGH when TX/RX complete; required by RadioLib |
| RST | ESP32 GPIO 4 | Reset — drive LOW 10ms then release to reset module |
| DIO1–DIO5 | Not connected | Not used in LoRa mode with RadioLib |
| ANT | 433 MHz whip antenna | Quarter-wave = 164 mm for 433 MHz; u.FL connector on RA-02 |

MicroSD SPI Adapter:

| SD Adapter Pin | Connect To | Notes |
| --- | --- | --- |
| VCC/3.3V | ESP32 3V3 | Use level-shifted adapter for 3.3V SD cards |
| GND | ESP32 GND | Common ground |
| SCK (CLK) | ESP32 GPIO 18 (SCK) | Shared VSPI bus with LoRa |
| MISO (DO) | ESP32 GPIO 19 (MISO) | Shared VSPI bus |
| MOSI (DI) | ESP32 GPIO 23 (MOSI) | Shared VSPI bus |
| CS | ESP32 GPIO 15 | Separate CS from LoRa — active LOW |

RCWL-0516 Radar Obstacle Sensor:

| RCWL Pin | Connect To | Notes |
| --- | --- | --- |
| VCC | ESP32 VIN/5V pin | Requires 5V — connect to DevKit 5V (USB VBUS) or vehicle 5V rail |
| GND | ESP32 GND | Common ground |
| OUT | ESP32 GPIO 34 | 3.3V digital output — GPIO34 is input-only, suitable here |
| CDS | Not connected | Light control pin — leave floating |

Status LED:

| LED Connection | Connect To | Notes |
| --- | --- | --- |
| LED Anode (+) | ESP32 GPIO 26 | Short pin of LED to resistor, then GPIO 26 |
| 330Ω Resistor | Between GPIO 26 and LED anode | Current limiting — do not omit |
| LED Cathode (–) | GND | Long pin of LED to GND |

# 4. DEVICE 2 — GROUND CONTROL UNIT (GCU): COMPLETE WIRING

The GCU is the operator's handheld interface. It consists of an ESP32 DevKit (LoRa communications + laptop bridge) and an Arduino Nano (display + joystick controller). They communicate via UART serial.

## 4.1 GCU ESP32 Pin Allocation

| GPIO Pin | Direction | Connected To | Signal | Notes |
| --- | --- | --- | --- | --- |
| GPIO 18 (SCK) | OUTPUT | LoRa SCK | SPI VSPI clock | Same pinout as PM for firmware reuse |
| GPIO 19 (MISO) | INPUT | LoRa MISO | SPI data in |  |
| GPIO 23 (MOSI) | OUTPUT | LoRa MOSI | SPI data out |  |
| GPIO 5 | OUTPUT | LoRa NSS/CS | LoRa chip select (active LOW) |  |
| GPIO 2 | INPUT | LoRa DIO0 | RX done interrupt | Required for RadioLib |
| GPIO 4 | OUTPUT | LoRa RST | LoRa hardware reset |  |
| GPIO 21 (SDA) | BIDIR | Arduino Nano A4 (SDA) | I2C optional bridge | Only if using I2C to Nano instead of UART |
| GPIO 22 (SCL) | OUTPUT | Arduino Nano A5 (SCL) | I2C optional bridge |  |
| GPIO 32 | INPUT (ADC) | Joystick X (VRX) | Analog 0–3.3V | ADC1_CH4 — connect joystick to 3.3V supply |
| GPIO 33 | INPUT (ADC) | Joystick Y (VRY) | Analog 0–3.3V | ADC1_CH5 |
| GPIO 25 | OUTPUT | LED Green (GPS lock) | Status indicator | 330Ω series resistor |
| GPIO 26 | OUTPUT | LED Amber (geofence) | Status indicator | 330Ω series resistor |
| GPIO 27 | OUTPUT | LED Red (obstacle/breach) | Alert indicator | 330Ω series resistor |
| GPIO 14 | INPUT | Button: Emergency Stop | Active LOW (pull-up) | 10kΩ pull-up to 3.3V; button to GND |
| GPIO 12 | INPUT | Button: Arm Geofence | Active LOW (pull-up) | 10kΩ pull-up to 3.3V |
| GPIO 13 | INPUT | Button: Disarm Geofence | Active LOW (pull-up) | 10kΩ pull-up to 3.3V |
| GPIO 1 (TX0) | OUTPUT | HC-05 RX / USB-Serial | UART0 JSON to laptop | Via voltage divider to HC-05 RX |
| GPIO 3 (RX0) | INPUT | HC-05 TX / USB-Serial | UART0 JSON from laptop | Direct connection from HC-05 TX (3.3V) |
| GPIO 16 (TX2) | OUTPUT | Arduino Nano RX0 | UART2 — display data to Nano | Level: 3.3V ESP32 TX → 1kΩ → Nano RX |
| GPIO 17 (RX2) | INPUT | Arduino Nano TX0 | UART2 — status from Nano | Nano TX is 5V — use 1kΩ+2kΩ voltage divider |

## 4.2 GCU Wiring Diagram — ASCII Schematic

╔══════════════════════════════════════════════════════════════════════════════╗

║          DEVICE 2 — GROUND CONTROL UNIT (GCU) — COMPLETE WIRING DIAGRAM    ║

╚══════════════════════════════════════════════════════════════════════════════╝

USB FROM LAPTOP                  ESP32 DevKit (38-pin)

─────────────              ┌───────────────────────────┐

5V USB ─── VIN ─────────── │ VIN          GPIO18(SCK)  │─── LoRa SCK

GND ─────────────────────  │ GND          GPIO19(MISO) │─── LoRa MISO

│              GPIO23(MOSI) │─── LoRa MOSI

3.3V OUT ──────────────    │ 3V3          GPIO5        │─── LoRa NSS/CS

(for sensors/buttons)      │              GPIO2        │─── LoRa DIO0

│              GPIO4        │─── LoRa RST

│                           │

│              GPIO32 (ADC) │─── Joystick VRX (X-axis)

│              GPIO33 (ADC) │─── Joystick VRY (Y-axis)

│                           │

│              GPIO25       │─── 330Ω ─── LED Green (GPS)

│              GPIO26       │─── 330Ω ─── LED Amber (Fence)

│              GPIO27       │─── 330Ω ─── LED Red  (Alert)

│                           │

BUTTON WIRING:             │              GPIO14       │─── 10kΩ→3V3 + BTN→GND (E-STOP)

3.3V ─── 10kΩ ─── GPIO     │              GPIO12       │─── 10kΩ→3V3 + BTN→GND (ARM)

Button ─── GND             │              GPIO13       │─── 10kΩ→3V3 + BTN→GND (DISARM)

│                           │

TO LAPTOP USB:             │              GPIO1 (TX0)  │─── HC-05 RX (via 1kΩ+2kΩ divider)

(via CH340 USB)            │              GPIO3 (RX0)  │─── HC-05 TX (direct 3.3V)

│                           │

TO ARDUINO NANO:           │              GPIO16(TX2)  │─── 1kΩ ─── Arduino Nano RX0

│              GPIO17(RX2)  │─── Arduino Nano TX0 (5V→divider)

└───────────────────────────┘

┌──────────────── ARDUINO NANO ────────────────────────┐

│  5V ─── VIN (from ESP32 5V or USB)                   │

│  GND ─── GND                                         │

│  RX0 ─── (from ESP32 GPIO16 via 1kΩ)                 │

│  TX0 ─── (to ESP32 GPIO17 via 1kΩ+2kΩ divider)       │

│  A4(SDA) ─── SSD1306 SDA                             │

│  A5(SCL) ─── SSD1306 SCL                             │

│  A0 ─── Joystick VRX   (optional: read here too)     │

│  A1 ─── Joystick VRY                                 │

└──────────────────────────────────────────────────────┘

┌──────── SSD1306 OLED DISPLAY ──────────────────┐

│  VCC ─── 3.3V (from Arduino Nano 3.3V pin)     │

│  GND ─── GND                                   │

│  SDA ─── Arduino Nano A4                       │

│  SCL ─── Arduino Nano A5                       │

│  I2C Address: 0x3C                              │

└────────────────────────────────────────────────┘

HC-05 BLUETOOTH:  VCC ─── 5V   GND ─── GND

3.3V DIVIDER:  ESP32 TX ─── 1kΩ ─┬─── HC-05 RX

└─── 2kΩ ─── GND   (gives ~2.2V at HC-05 RX — safe)

JOYSTICK (KY-023):  VCC ─── 3.3V   GND ─── GND

VRX ─── GPIO32 (ADC)   VRY ─── GPIO33 (ADC)   SW ─── GPIO(optional, 10kΩ pull-up)

## 4.3 GCU Arduino Nano Pin Allocation

| Nano Pin | Direction | Connected To | Notes |
| --- | --- | --- | --- |
| VIN / +5V | POWER IN | ESP32 5V pin or USB 5V | Powers Nano; also supplies OLED via 3.3V out |
| GND | GROUND | ESP32 GND (common) | Shared ground — essential |
| RX0 (D0) | INPUT | ESP32 GPIO16 (TX2) via 1kΩ | Receives display update strings from ESP32 |
| TX0 (D1) | OUTPUT | ESP32 GPIO17 (RX2) via 1kΩ+2kΩ | 5V TX — must be divided before ESP32 RX (3.3V) |
| A4 (SDA) | BIDIR | SSD1306 SDA | I2C data to OLED |
| A5 (SCL) | OUTPUT | SSD1306 SCL | I2C clock to OLED |
| A0 | INPUT (ADC) | Joystick VRX | 10-bit ADC — center ~512 |
| A1 | INPUT (ADC) | Joystick VRY | 10-bit ADC — center ~512 |

# 5. DEVICE 3 — GPS SPOOFER/VALIDATOR (SV): COMPLETE WIRING

Device 3 is a UART-level NMEA injector. It connects directly by wire to the NEO-6M module on Device 1 (PM), bypassing RF entirely. It generates synthetic GPS NMEA sentences to test geofence boundary response. This approach is fully legal — no RF signals are emitted.

## 5.1 SV Pin Allocation

| GPIO Pin | Direction | Connected To | Signal | Notes |
| --- | --- | --- | --- | --- |
| GPIO 17 (TX2) | OUTPUT | PM NEO-6M RX pin (via 3-wire cable) | UART2 — synthetic NMEA at 9600 baud | This overrides GPS satellite data on the PM |
| GPIO 16 (RX2) | INPUT | PM NEO-6M TX pin (monitor) | Read back NMEA being parsed by PM | Optional — used to verify injection is working |
| GPIO 21 (SDA) | BIDIR | SSD1306 SDA | I2C data to local OLED | Shows currently injected coordinates |
| GPIO 22 (SCL) | OUTPUT | SSD1306 SCL | I2C clock to local OLED |  |
| GPIO 0 | INPUT | Tactile button (to GND) | Cycle through pre-stored test coordinates | 10kΩ pull-up to 3.3V; boot pin — OK for input |
| GPIO 2 | OUTPUT | Status LED via 330Ω | Blinks when spoofing is active |  |
| GPIO 1 (TX0) | OUTPUT | USB / HC-05 RX | UART0 — receive spoof coords from laptop | For live manual coordinate injection |
| GPIO 3 (RX0) | INPUT | USB / HC-05 TX | UART0 — laptop sends target lat/lon |  |
| 3V3 | POWER OUT | SSD1306 VCC | 3.3V for OLED |  |
| VIN/5V | POWER IN | USB 5V or 3× AA + 7805 | 5V supply for SV device |  |

## 5.2 SV Wiring Diagram — ASCII Schematic

╔══════════════════════════════════════════════════════════════════════════════╗

║        DEVICE 3 — GPS SPOOFER / VALIDATOR (SV) — COMPLETE WIRING DIAGRAM   ║

╚══════════════════════════════════════════════════════════════════════════════╝

USB POWER                    ESP32 DevKit (Device 3 — SV)

─────────             ┌──────────────────────────────┐

5V ─── VIN ─────────  │ VIN           GPIO17(TX2)    │──┐ TO PM (Device 1)

GND ─────────────────  │ GND           GPIO16(RX2)    │──┤ 3-wire cable to

│               GPIO21(SDA)    │──┤ NEO-6M on PM

3.3V internal ──────   │ 3V3           GPIO22(SCL)    │──┘

│                              │

OLED DISPLAY:          │  GPIO21(SDA) ─── SSD1306 SDA │

SSD1306 ─── Nano A4    │  GPIO22(SCL) ─── SSD1306 SCL │

│                              │

BUTTON:                │  GPIO0 ──── 10kΩ→3.3V       │

GPIO0 ─── BTN ─── GND  │             + BTN → GND      │

│                              │

STATUS LED:            │  GPIO2 ─── 330Ω ─── LED ─── GND │

│                              │

LAPTOP / HC-05:        │  GPIO1(TX0) ─── HC-05 RX    │

(receive coords)       │  GPIO3(RX0) ─── HC-05 TX    │

└──────────────────────────────┘

┌──── 3-WIRE CONNECTION: SV (Device 3) → PM (Device 1) ───────────────────┐

│                                                                          │

│  SV ESP32 GPIO17 (TX2) ─────────────────► PM NEO-6M RX pin             │

│  SV ESP32 GPIO16 (RX2) ◄───────────────── PM NEO-6M TX pin  (optional) │

│  SV GND               ◄────────────────── PM GND  (MUST share ground)  │

│                                                                          │

│  Cable: 3-wire dupont or twisted pair, max length ~50 cm                │

│  IMPORTANT: PM's ESP32 GPIO17 (TX2 to NEO-6M RX) must be DISCONNECTED  │

│  when SV is injecting — only ONE device should drive NEO-6M RX pin.    │

└──────────────────────────────────────────────────────────────────────────┘

⚠ INJECTION PROCEDURE:

1. Power OFF PM's ESP32 GPIO17 drive (disconnect or set GPIO17 to input)

2. Connect SV GPIO17 → PM NEO-6M RX

3. Connect SV GND → PM GND

4. Power ON SV — it will now feed NMEA at 9600 baud

5. PM GPS will process synthetic coordinates as if real satellite fix received

# 6. POWER DISTRIBUTION & PROTECTION

## 6.1 Device 1 (PM) Power Tree

Vehicle 5V BEC (or USB 5V) ─── 500mA Polyfuse ─── PM VIN

│

├─── ESP32 AMS1117-3.3V ─── 3.3V Rail ─────────────────────────────┐

│                                 │                                 │

│                          NEO-6M VCC (45mA)              LoRa VCC (120mA pk)

│                          MPU-6050 VCC (4mA)             MicroSD VCC (100mA pk)

│                          BMP280 VCC (3mA)               ESP32 Core (240mA pk WiFi)

│

└─── 5V Rail (direct) ─── RCWL-0516 VCC (3mA)

└─ ESP32-CAM 5V   (310mA pk)

Recommended: 100µF electrolytic + 100nF ceramic bypass capacitors on 3.3V rail

Recommended: 100µF electrolytic on 5V input rail near ESP32 VIN

## 6.2 Power Budget Summary

| Component | Voltage | Typical (mA) | Peak (mA) | Rail |
| --- | --- | --- | --- | --- |
| ESP32 DevKit | 3.3V | 80 | 240 | 3.3V |
| NEO-6M GPS | 3.3V | 45 | 45 | 3.3V |
| MPU-6050 | 3.3V | 4 | 4 | 3.3V |
| GY-BMP280 | 3.3V | 3 | 3 | 3.3V |
| LoRa SX1278 | 3.3V | 15 (RX) | 120 (TX) | 3.3V |
| MicroSD | 3.3V | 25 | 100 | 3.3V |
| Status LED | 3.3V | 10 | 10 | 3.3V |
| RCWL-0516 | 5V | 3 | 3 | 5V direct |
| ESP32-CAM | 5V | 180 | 310 | 5V direct |
| TOTAL | 5V input | ~365 avg | ~835 peak | 5V @ 2A supply |

⚠  Use a 500mA polyfuse (resettable fuse) on the 5V input line of the PM. If drawing power from a UAV BEC, verify it is rated for at least 2A continuous. Voltage sag under load can cause GPS/LoRa reset events.

## 6.3 Decoupling Capacitor Placement

| Location | Capacitor | Purpose |
| --- | --- | --- |
| 3.3V rail — near ESP32 3V3 pin | 100µF electrolytic + 100nF ceramic | Stabilise 3.3V under load spikes (LoRa TX, WiFi burst) |
| 5V input — near VIN pin | 100µF electrolytic | Filter BEC/USB supply ripple |
| MPU-6050 VCC to GND | 100nF ceramic | Local decoupling — reduce I2C noise pickup |
| BMP280 VCC to GND | 100nF ceramic | Local decoupling |
| LoRa RA-02 VCC to GND | 10µF tantalum + 100nF ceramic | Critical — LoRa TX draws 120mA spikes |
| NEO-6M VCC to GND | 10µF electrolytic + 100nF ceramic | Prevent reset during cold start current surge |

# 7. PHYSICAL ASSEMBLY & INTEGRATION NOTES

## 7.1 I2C Bus Guidelines

Both MPU-6050 and BMP280 share the same I2C bus (SDA/SCL). Their addresses are different (0x68 and 0x76 respectively) so there is no conflict. Key assembly rules:

| Rule | Detail |
| --- | --- |
| Pull-up resistors | Only ONE set of 4.7kΩ pull-ups needed per bus. GY-521 and GY-BMP280 breakouts both have onboard pull-ups — if both are present, remove the pull-up resistors from one board (desolder or cut trace) to avoid parallel 2.35kΩ which may cause issues at higher speeds. |
| Wire length | Keep I2C wires under 10 cm. Longer runs increase capacitance and cause signal integrity issues at 400 kHz. |
| Cable routing | Keep I2C traces away from SPI, UART, and LoRa antenna lines. |
| Verification | Use I2C scanner sketch to confirm both 0x68 and 0x76 are detected before soldering. |

## 7.2 SPI Bus Guidelines

LoRa (RA-02) and MicroSD share the VSPI SPI bus. Each has its own CS (chip select) line.

| Rule | Detail |
| --- | --- |
| CS management | Only ONE CS line is driven LOW at a time. Firmware must ensure LoRa CS is HIGH when accessing SD, and SD CS is HIGH when accessing LoRa. RadioLib and SD.h handle this automatically when given correct CS pins. |
| SPI speed | Use 8 MHz for SD (reliable on wires/breadboard). LoRa SPI can run at 8–10 MHz. Set in firmware. |
| Wire length | Keep SPI wires under 10 cm on breadboard. On PCB, use 50Ω controlled impedance traces. |
| LoRa RA-02 note | RA-02 has exposed metal shield — if mounting on foam/acrylic, no issue. Keep away from GPS antenna (min 10 cm). |

## 7.3 Antenna Placement Rules

| Antenna | Rule | Reason |
| --- | --- | --- |
| NEO-6M GPS patch antenna | Mount with clear sky view. No metal above it within 5 cm. Orient flat side up. | Patch antenna is directional — metal above it attenuates the signal severely. |
| LoRa 433 MHz whip (17 cm) | Orient vertically (perpendicular to ground). Keep 10 cm away from GPS antenna. | LoRa is omnidirectional when vertical. Horizontal reduces gain by up to 20 dB in vertical direction. |
| ESP32-CAM PCB WiFi antenna | Keep antenna end (the notch end of module) unobstructed. No metal within 2 cm. | PCB antenna is integrated — nearby metal detunes it significantly. |
| HC-05 Bluetooth antenna | Module has internal chip antenna — keep 1 cm clearance all sides. | Short range (10 m) so less critical, but still avoid grounded metal nearby. |

## 7.4 GPIO Protection Recommendations

| Protection | Where to Apply | Component |
| --- | --- | --- |
| Series resistor on all GPIO outputs | LED, status signals, UART TX lines | 100Ω – 330Ω in series protects ESP32 if accidentally shorted |
| Pull-up on button inputs | All tactile buttons | 10kΩ to 3.3V; button connects GPIO to GND (active LOW) |
| Voltage divider on 5V UART signals | HC-05 TX → ESP32 RX; Arduino Nano TX → ESP32 RX | 1kΩ + 2kΩ divider gives 3.3V from 5V signal |
| ESD protection diode | LoRa DIO0, GPS UART | BAT54S or 1N4148 from GPIO to GND; optional but best practice |
| Polyfuse on 5V input | PM main supply line | 500mA – 1A polyfuse (resettable) prevents vehicle battery damage on short |

## 7.5 PM Mechanical Mounting Checklist

| Component | Mounting Requirement | Why |
| --- | --- | --- |
| NEO-6M GPS | Mount as high as possible; clear sky view; no metal above it | GPS patch antenna requires unobstructed sky — even a carbon fiber frame above it attenuates signal |
| MPU-6050 IMU | Mount rigidly to the vehicle frame — no vibration dampening foam | Foam isolation introduces phase lag in vibration data; need true vehicle dynamics for accurate attitude |
| RCWL-0516 Radar | Face in direction of travel; copper tape shield on 3 sides | Makes sensor directional (forward-facing cone) instead of omnidirectional; prevents propeller false triggers |
| LoRa antenna | Vertical (pointing up/down); minimum 10 cm from GPS antenna | Vertical orientation = omnidirectional coverage; proximity to GPS causes desensitization |
| ESP32-CAM | Forward or downward depending on mission; WiFi end unobstructed | Clear field of view for camera; WiFi signal should not be blocked by frame carbon |
| MicroSD adapter | Anywhere accessible; SPI wires < 10 cm | Short SPI traces for signal integrity; accessible for card swap between missions |

# 8. HARDWARE BRING-UP & VERIFICATION SEQUENCE

Follow this sequence in order. Do NOT power all sensors simultaneously until each one is individually verified. This prevents damage from incorrect wiring.

| Step | Action | Tool / Method | Pass Criteria |
| --- | --- | --- | --- |
| 1 | Inspect wiring — before any power | Visual check + multimeter continuity test | No shorts between VCC/GND rails; all connections present |
| 2 | Power ESP32 alone (no sensors connected) | Multimeter on 3V3 pin | 3.28V – 3.32V stable at 3V3 pin; LED on board blinks |
| 3 | Connect MPU-6050; run I2C scanner | Arduino I2C scanner sketch | Address 0x68 detected; no 'unknown' or timeout |
| 4 | Connect BMP280; re-run I2C scanner | Arduino I2C scanner sketch | Both 0x68 AND 0x76 detected; no conflicts |
| 5 | Flash PM firmware; open Serial Monitor | Arduino IDE / PlatformIO | Boot messages show: GPS OK, IMU OK, BARO OK |
| 6 | Connect GPS antenna; place near window | Serial Monitor — watch NMEA | NMEA sentences appear; satellites > 4 within 5 min; position fix shown |
| 7 | Verify IMU: tilt PM 45° | Serial Monitor | Roll/pitch values change correctly; ±1° accuracy at rest after calibration |
| 8 | Verify BMP280: breathe on sensor gently | Serial Monitor | Altitude reading changes momentarily then returns to baseline |
| 9 | Wave hand at RCWL-0516 (within 1 m) | Serial Monitor | Obstacle flag goes HIGH within 200 ms; stays HIGH for ~2 seconds |
| 10 | Connect LoRa; verify init | Serial Monitor | LoRa init OK message; no SPI errors |
| 11 | Power GCU; verify LoRa link between PM and GCU | GCU OLED + Laptop Serial | GCU OLED shows incoming telemetry within 5 seconds; JSON on laptop at 4 Hz |
| 12 | Insert microSD card (FAT32 formatted); verify logging | Serial Monitor + card reader | LOG_Task starts; CSV file created on card; rows written every 500 ms |
| 13 | Test geofence: upload circle from laptop, arm, inject breach via SV | Laptop software + Device 3 SV | GCU shows breach alert; PM executes configured action; SD log shows breach event |

## 8.1 Common Hardware Faults & Fixes

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| I2C scanner shows no devices | Pull-up resistors missing or two sets in parallel (< 2kΩ) | Check 4.7kΩ pull-ups present; remove duplicate set from one breakout board |
| GPS never gets fix (no NMEA) | UART2 not started or wrong GPIO | Verify Serial2.begin(9600, SERIAL_8N1, 16, 17) in firmware; check wiring GPIO16↔NEO TX |
| LoRa init fails / SPI error | CS pin conflict or 5V on LoRa VCC | Verify LoRa VCC is 3.3V ONLY; check GPIO5 is CS for LoRa; recheck SPI pin assignments |
| RCWL-0516 always HIGH / false triggers | 5V not connected; propeller interference; no debounce | Confirm 5V supply to RCWL; add copper shield; implement 5-sample software debounce |
| BMP280 reads 0 or garbage | SDO/CSB pins floating or wrong I2C address | Tie SDO to GND (address 0x76) and CSB to 3.3V; verify Adafruit BMP280 lib uses 0x76 |
| MicroSD init fails | Wrong CS pin; SD card not FAT32; SPI speed too high | Confirm GPIO15 is SD CS; format card FAT32; set SPI speed to 4 MHz in SD.begin() |
| GCU OLED blank | I2C address wrong or Arduino Nano not powered | Scan for 0x3C; check 5V to Nano; verify SSD1306 library address matches hardware |
| No LoRa link PM↔GCU | Sync word mismatch or frequency mismatch | Both must be 433.0 MHz, SF9, BW125, sync word 0xA5 — verify both firmware files match |
| ESP32-CAM no WiFi stream | Wrong board config or power supply too weak | Select 'AI Thinker ESP32-CAM' in Arduino IDE; ensure 5V @ 500mA dedicated supply |

# 9. COMPLETE HARDWARE CROSS-REFERENCE

Full inventory of all components across all three devices including quantities, status, and connection summary.

| Component | Model | Qty Available | Qty Used | Device(s) | Interface | Status |
| --- | --- | --- | --- | --- | --- | --- |
| ESP32 DevKit | ESP32-D0WDQ6 38-pin | 3 | 3 | D1, D2, D3 | Central MCU | ACTIVE |
| NEO-6M GPS | u-blox NEO-6M-0-001 | 2 | 1 | Device 1 (PM) | UART2 @ 9600 baud | 1 ACTIVE, 1 SPARE |
| IMU | MPU-6050 GY-521 | 2 | 1 | Device 1 (PM) | I2C @ 0x68 | 1 ACTIVE, 1 SPARE |
| Barometer | GY-BMP280 | 2 | 1 | Device 1 (PM) | I2C @ 0x76 | 1 ACTIVE, 1 SPARE |
| Radar Sensor | RCWL-0516 | 2 | 1 | Device 1 (PM) | Digital GPIO34 | 1 ACTIVE, 1 SPARE |
| Camera | ESP32-CAM OV2640 | 1 | 1 | Device 1 (PM) | WiFi AP (standalone) | ACTIVE |
| LoRa Radio | SX1278 RA-02 433 MHz | 2 | 2 | Device 1 + 2 | SPI VSPI | 2 ACTIVE |
| MicroSD Adapter | SPI SD module | 1 | 1 | Device 1 (PM) | SPI VSPI (shared) | ACTIVE |
| Bluetooth Module | HC-05 (CSR BC417) | 1 | 1 | Device 2 (GCU) | UART @ 115200 | ACTIVE |
| Arduino Nano | ATmega328P 16MHz | 1 (of 4) | 1 | Device 2 (GCU) | UART to ESP32 | ACTIVE |
| OLED Display | 0.96" SSD1306 I2C | 2 | 1–2 | Device 2 (GCU) | I2C @ 0x3C | 1+ ACTIVE |
| Joystick | KY-023 2-axis | 1 | 1 | Device 2 (GCU) | ADC GPIO32/33 | ACTIVE |
| Push Buttons | Tactile momentary | 4+ | 3–4 | Device 2 (GCU) | Digital GPIO (pull-up) | ACTIVE |
| Status LEDs | 5mm LED (R/G/A) | 5+ | 4 | D1 (×1), D2 (×3) | Digital GPIO + 330Ω | ACTIVE |
| HW-130 Motor Shield | L298P Motor Driver | 1 | 0 | NOT USED | — | SPARE |
| FS-iA6B RC Receiver | Flysky iBUS receiver | 1 | 0 (optional) | Optional on D1 | iBUS/PPM GPIO | OPTIONAL |
| HW-837 Relay Module | 5V relay module | 1 | 0 (optional) | Optional on D1 | Digital GPIO | OPTIONAL |
| Servo Motor | Standard 5V servo | 1 | 0 | NOT USED | — | SPARE |
| Arduino UNO/Mega (×3 extra) | ATmega328P/2560 | 3 | 0 | NOT USED | — | SPARE |
| 2nd NEO-6M GPS | u-blox NEO-6M | 1 | 0 | Not needed for SV | — | SPARE |

# 10. QUICK REFERENCE CARDS

## 10.1 ESP32 Pin Quick Reference — Device 1 (PM)

| GPIO | Function | GPIO | Function |
| --- | --- | --- | --- |
| GPIO 16 (RX2) | NEO-6M TX — NMEA IN | GPIO 5 | LoRa CS (SPI) |
| GPIO 17 (TX2) | NEO-6M RX — UBX OUT | GPIO 15 | SD CS (SPI) |
| GPIO 21 (SDA) | I2C: MPU-6050 + BMP280 | GPIO 2 | LoRa DIO0 IRQ |
| GPIO 22 (SCL) | I2C: MPU-6050 + BMP280 | GPIO 4 | LoRa RST |
| GPIO 18 (SCK) | SPI VSPI: LoRa + SD | GPIO 34 | RCWL-0516 OUT (obstacle) |
| GPIO 19 (MISO) | SPI VSPI: LoRa + SD | GPIO 26 | Status LED + 330Ω |
| GPIO 23 (MOSI) | SPI VSPI: LoRa + SD | GPIO 1/3 | UART0: USB/HC-05 |

## 10.2 I2C Device Address Summary

| Device | I2C Address | Address Selection Pin | Used On |
| --- | --- | --- | --- |
| MPU-6050 (GY-521) | 0x68 | AD0 pin = GND (default) | Device 1 PM |
| GY-BMP280 | 0x76 | SDO pin = GND (default) | Device 1 PM |
| SSD1306 OLED | 0x3C | SA0 pin = GND (default) | Device 2 GCU (via Arduino Nano) |
| SSD1306 OLED (SV) | 0x3C | SA0 pin = GND (default) | Device 3 SV |

## 10.3 LoRa Configuration Parameters (Both PM and GCU must match)

| Parameter | Value | Rationale |
| --- | --- | --- |
| Frequency | 433.0 MHz | ISM band — legal in India for unlicensed use |
| Spreading Factor | SF9 | Balance of ~1–2 km range and ~1.76 kbps data rate |
| Bandwidth | 125 kHz | Standard — best noise immunity for field use |
| Coding Rate | 4/6 | Error correction for RF noise in outdoor environments |
| TX Power | +17 dBm (~50 mW) | Long range without excessive battery consumption |
| Sync Word | 0xA5 | Custom — filters out other LoRa devices on 433 MHz |
| Preamble Length | 8 symbols | Standard; increase to 12 in high-noise RF environments |
| CRC | Enabled | Mandatory for packet integrity detection |
| Telemetry Packet Size | 24 bytes | PM → GCU at 4 Hz = 96 bytes/s (well within link capacity) |
| Command Packet Size | Up to 64 bytes | GCU → PM on-demand (geofence upload, commands) |

## 10.4 Firmware Library Quick Reference

| Library | Install Method | Used On | Purpose |
| --- | --- | --- | --- |
| TinyGPS++ | Arduino Library Manager | PM (ESP32) | Parse NMEA from NEO-6M |
| RadioLib | Arduino Library Manager | PM + GCU (ESP32) | Full SX1278 LoRa driver with interrupt support |
| Adafruit BMP280 | Arduino Library Manager | PM (ESP32) | BMP280 pressure/altitude |
| MPU6050 (I2Cdevlib) | Library Manager / GitHub | PM (ESP32) | MPU-6050 accelerometer/gyroscope |
| ArduinoJson | Arduino Library Manager | GCU (ESP32) | Parse/generate JSON for laptop bridge |
| Adafruit SSD1306 + GFX | Arduino Library Manager | GCU (Arduino Nano) | OLED display driver |
| SD.h | Built-in with ESP32 core | PM (ESP32) | MicroSD data logging |
| Wire.h | Built-in | PM + GCU | I2C for all sensors |
| SPI.h | Built-in | PM + GCU | SPI for LoRa and SD |
| Preferences.h | ESP32 built-in (NVS) | PM (ESP32) | Save IMU calibration offsets to flash |
| AsyncWebServer | Arduino Library Manager | PM (ESP32) | Serve ESP32-CAM MJPEG stream page over WiFi |

# 11. COMPONENT REFERENCES & DATASHEETS

| # | Document | Source |
| --- | --- | --- |
| 1 | u-blox NEO-6M Hardware Integration Manual, GPS-13C Rev.1 | u-blox AG (2011) — docs.u-blox.com |
| 2 | u-blox Protocol Specification UBX-13003221 | u-blox AG (2018) — docs.u-blox.com |
| 3 | InvenSense MPU-6050 Product Specification Rev. 3.4 | InvenSense Inc. (2013) — invensense.tdk.com |
| 4 | Bosch BMP280 Datasheet v1.27 | Bosch Sensortec GmbH (2019) — bosch-sensortec.com |
| 5 | Semtech SX1276/77/78/79 Datasheet Rev 7 | Semtech Corporation (2019) — semtech.com |
| 6 | Semtech Application Note AN1200.22 — LoRa Modulation Basics | Semtech (2015) — semtech.com |
| 7 | Espressif ESP32 Technical Reference Manual v5.2 | Espressif Systems (2024) — espressif.com |
| 8 | RCWL-0516 Microwave Sensor Module Datasheet | RCW Technology Co. — github.com/jdesbonnet/RCWL-0516 |
| 9 | AI-Thinker RA-02 LoRa Module Datasheet | AI-Thinker — ai-thinker.com |
| 10 | AI-Thinker ESP32-CAM Module Specification | AI-Thinker — ai-thinker.com |
| 11 | CSR BC417 Bluetooth Module Datasheet (HC-05) | Cambridge Silicon Radio / Microchip |
| 12 | Atmel ATmega328P Datasheet | Microchip Technology (Arduino Nano) — microchip.com |
| 13 | Solomon Systech SSD1306 OLED Controller Datasheet | Solomon Systech Ltd — solomon-systech.com |

| Prepared by | Asad Sayyed |
| --- | --- |
| Institution | Vishwakarma Institute of Technology, Pune |
| Document | GNSS Geofencing System — Complete Hardware Implementation Reference |
| Date | April 2026 |